# Diploma Lefel 2 Gosod Brics - Rhestr Termau

# Level 2 Diploma in Bricklaying Bilingual Glossary of Terms

Defnyddir y geiriau allweddol a’r termau canlynol yn yr unedau:

The following key words and terms are used in the units:

| Term Saesneg / English term | Term Cymraeg / Welsh Term | Diffiniad / Definition |
| --- | --- | --- |
| Band Course | Cwrs band | One or more courses forming a decorative opening |
| Barge-Board | Astell dywyll | A board covering the ends of the horizontal timbers of a roof which would otherwise be exposed at the gable end. |
| Bat | Bat | Part of a brick greater than one-quarter |
| Beam-Filling | Bricwaith a osodir i lanw rhwng diwedd y distiau neu drawstiau | The brickwork between the ends of joists or beams |
| Bearing | Cynhaliad | The amount by which a piece of construction rests upon its support |
| Bridged Cavity | Pontio ceudod | Something that touches both skins of a cavity wall, i.e. lintel, tie wire, cavity liner, mortar droppings |
| Bulking | Swmpuso | Occurs particularly in wet sand and ballast when wet but before saturation |
| Cantilever | Cantilifer | A beam built into a wall and held down at one end and unsupported at the other |
| Cap or Capping | Rhoi cap neu Gapio | The top cover of a pier, wall, or chimney stack. |
| Cast-in-situ | Wedi’i gastio/fowldio yn y fan a’r lle | Moulded in the position it will occupy permanently |
| Cavity Tray | Hambwrdd ceudod | Membrane placed over a bridged cavity, see bridged cavity |
| Center | Canolbwynt neu graidd crymedd | A temporary frame shaped to contour for supporting an arch or other feature during its construction, being removed when setting has taken place |
| Chase | Torri rhych | A narrow recess cut in the brickwork (i.e. to take conduit or piping). |
| Coping | Copin | The weathering feature on top of an external wall |
| Coring Hole | Twll Gwagio | A temporary hole left in a cavity wall at ground level to enable mortar droppings to be removed from cavity. |
| Creasing | Bargodion teils | Usually two courses of tiles built in beneath a coping. |
| Efflorescence | Ewlychiad/ffwng gwyn | A white deposit which may form on the surface of new bricks if the latter contain a high proportion of mineral salts. |
| Expanded Metal | Metel ehangedig/ffurf diemwnt | Metal reinforcement made out of sheet metal to form a mesh. |
| Feathered Edged Coping | Copin Min Main/Min Pluen | A coping that is thinner at one edge than the other. |
| Flaunching | Llain oleddol o sment o gwmpas corn simnai | The cement fillet at the junction around a chimney pot. |
| Gables | Talcen tŷ | The part of the wall in the triangle formed by the sloping sides of the roof. |
| Head | Pen uchaf | Horizontal top member of a door window frame |
| Honeycombed Wall | Wal grwybrol/ridyllog | A wall with bricks set so as to provide openings at regular intervals to give ventilation through the wall. Used to support ground floor joists. |
| Horns | Cyrn | The projections on a door head for building into the joints. |
| Indent | Bwlch cilfachog | A recess formed in the brickwork. |
| Jamb | Ystlysbost | The brickwork on either side of an opening. |
| Lintel | Lintel/Capan drws | A horizontal member for spanning an opening to support the structure above. |
| Overhand Work | Gwaith tros law | Facework executed from the back of the wall. |
| Oversailing | Bargodi | Projecting from the general face of the wall |
| Parapet Wall | Wal barapet | A low Wall on the edge of a building or a platform. |
| Party Wall | Wal gydrannol | The dividing wall between adjoining buildings |
| Insulation | Deunydd inswleiddio | Materials used to retain heat and improve the thermal value of a building, may also be used for managing sound transfer |
| Joggle | Uniad deintiog | Cavity into which grout is poured, often to form a joint |
| Jointing | Uniadu | As work proceeds. |
| Mid Girth | Mesuryn canol | Often referred to as a centre line calculation, used for calculating quantities of materials |
| Movement Joint | Uniad symudiad | A joint to allow for the co-efficient of thermal expansion in materials enabling expansion and contraction. |
| Padstone | Carreg bad | A masonry unit incorporated in a structure to help spread the load through a wall |
| Piers | Pileri | Brickwork used for support in walls or as pillars, attached and detached. |
| Plinth | Plinth | The projecting base of a wall or column |
| Pointing | Pwyntio | At a later date. |
| Pre-cast | Rhag-gastiedig | Made-up beforehand |
| Profiles | Byrddau proffil | Often same as “Templet”. Boards fixed horizontally to ground pegs at the ends of a wall before construction commences in order that lines may be stretched across to mark the position of the wall |
| Queen-Closer | Caefa Brenhines | Part of a brick, cut to 46 mm on face. |
| rusticated | rwstig | A blocking proud of the wall face, usually several courses of bricks followed by one or two flush, commonly used on quoins |
| Reinforced Brickwork | Bricwaith cyfnerth | Brickwork strengthened by inserting reinforcement between the mortar joints. |
| Reinforced Concrete | Concrit Cyfnerth | Concrete containing metal rods or steel mesh to give additional strength. |
| Retaining Wall | Wal gynhaliol | A wall built for the purpose of keeping in position a volume of earth or liquid. |
| Reveal | Cil | The portion of a wall at the side of a window or door opening when the jambs are recessed or rebated. |
| Sleeper Walls | Waliau cynnal | Dwarf walls erected at intervals between the main walls to provide intermediate supports to ground floor joists, usually built honeycombed. |
| Saddle-back | Copin trumiog | A pointed coping, i.e. with sloping sides. |
| Screed | Sgrîd | A layer of small aggregate concrete approx.. 50 mm thick laid on top of concrete floor to provide a smooth, level floor finish. |
| Solid Walls | Waliau Solet | Walls of a thickness of 1 brick and greater |
| Soldier Course | Cwrs milwrio | Course of bricks laid on end. |
| String Course | Llin-gwrs | A long narrow course projecting from the general face of the brickwork |
| Struck | Gwasgu mewn gan ddefnyddio ymyl y trywel | Refers to the mortar in pointing work being pressed inwards with the edge of the trowel. |
| Tensile Forces | Grymoedd Tynnol | A measure of the ability of material to resist a force that tends to pull it apart. |
| Tile Creasing | Bargodion Teils | The use of flat tiles as a decorative course |
| Timbering | Gwaith coed cynhaliol | The operation of supporting earth in trench work, etc., with heavy timbers. |
| Toothing | Danheddu | Leaving the vertical end of a wall unfinished in its bond to enable the wall to be continued at a later stage. |
| Wall Plate | Walblad | A timber bedded on the top of a wall for supporting joists or rafters. |
| Weep Holes | Tyllau Diferu | Small openings in a wall to permit the escape of water from the back of the wall. |
| Withes | Gwdenni | The thin division between adjoining flue liners, sometimes termed mid-feathers. |